Supplementary file

# A. Supplementary formula

The following presents the steps for calculating indicator weights using the G1-CRITIC method, and the steps for evaluating the operational schemes of WF-FJOS with the TOPSIS method. In the evaluation matrix X, m represents the number of operation schemes, and n represents the number of evaluation indicators.

(1) Determination of subjective weights based on G1 method

The G1 method is a subjective weighting approach. The calculation steps are as follows 1:

1) Establish the sequence of indicators

(S1)

where G is the evaluation index set, indicator g1 is the most important and gn​ is considered the least significant.

2) Determine the relative importance of adjacent indicators

(S2)

where rk represents the ratio of importance between adjacent indicators, k=2, 3,…n.

3) Calculate the subjective weight coefficients

(S3)

(S4)

where ωord,k denotes the weight of indicator gk.

(2) Determination of objective weights based on CRITIC method

The CRITIC method is an objective weighting method that determines weights based on the variability of each indicator across different evaluation schemes and the conflict between different indicators. The specific steps are as follows 2:

1) Normalization of the evaluation matrix

Since the indicators have different dimensions, dimensionless processing is required to derive the normalized matrix X'=[xij']m×n. The methods for handling positive and negative indicators can be expressed as:

(S5)

(S6)

where xij is the jth indicator for scheme i, max(xj) and min(xj) denote the maximum and minimum values of the jth indicator, respectively.

2) Calculate the standard deviation and correlation coefficient

In the CRITIC method, the standard deviation is used to quantify variability, and the conflict between indicators is reflected by correlation coefficients. The standard deviation and correlation coefficient of the normalized evaluation matrix X' are calculated as follows:

(S7)

(S8)

where xj' is the mean value of indicator j, ξj is the standard deviation of indicator j, ρsj is the correlation coefficient between indicator s and indicator j, Xs' and Xj' are the sth and jth columns of the matrix X', respectively.

3) Calculate the information content for each indicator

(S9)

Here s=1n(1-ρsj) represents the conflict of indicator j, Gj is the information content of indicator j.

4) Calculate the objective weight coefficients

(S10)

where ωcri,j is the objective weight of indicator j obtained by the CRITIC method.

(3) Determination of comprehensive weights

To balance subjective and objective weights, this paper employs the principle of minimum discrimination information 3 to calculate the comprehensive weights for each indicator:

(S11)

where ωj is the comprehensive weight of indicator j.

(4) Evaluation method of TOPSIS

After obtaining the comprehensive weights for all evaluation indicators, the alternative schemes of the WF-FLJOS can be assessed. The TOPSIS method is a comprehensive evaluation approach that ranks schemes based on their relative proximity to the ideal solution. The calculation steps of TOPSIS method are as follows 2:

1) Construct the weighted normalized matrix

By multiplying the normalized matrix X' with the comprehensive weights determined by the G1-CRITIC method, the weighted normalized matrix V=[vij]m×n is obtained:

(S12)

2) Calculate relative distances

The optimal and worst values of indicators are used to construct the positive and negative ideal solution vectors:

(S13)

The euclidean distance of each scheme to the positive and negative ideal solutions can be written as:

(S14)

where V+ and V- represent the positive and negative ideal solution vectors, respectively, max(Vj) and min(Vj) represent the maximum and minimum values ​​of indicator j, respectively, Vj+ and Vj - represent the jth elements of the positive and negative ideal solutions, respectively, Di+ and Di- are the distance of scheme i to the positive and negative ideal solutions, respectively.

3) Calculate the relative proximity of each evaluation scheme to the optimal scheme

The relative proximity of each evaluation scheme to the ideal solutions can be expressed as:

(S15)

where Si is the relative proximity of evaluation scheme i.

# B. Supplementary figure and parameters

Figure 15 shows wind power output scenarios.

FIG. 15. Wind power output scenarios.

As shown in Eq. (11) of the main text, the electricity price for main grid users is primarily determined by the power purchase costs and reserve capacity costs paid by the power grid to power generation entities. For the sake of explanation, this paper assumes that the power generation side includes the wind farm and thermal power units. Therefore, Eq. (11) can be written as:

(S16)

where cg,us is the electricity price indicator for grid-side users, Ctw,e and Cttp,e are the total electricity sales prices of the wind farm and thermal units in the energy market, respectively, Ctg,ad is the reserve cost paid by the main grid to balance the uncertainty of wind power output, Ptg,us is the power consumption of grid-side users, ∆t is the time interval, Tc is the total evaluation time period.

The load forecast data from Elia 4 are used and converted to the required scale. The unit reserve capacity compensation price for thermal power units is $26.6/MWh. To balance wind power output deviations, thermal power units need to adjust their output in the balancing market. The upward and downward adjustment prices for thermal units are set at 1.3 and 0.7 times the grid-connected electricity price in the day-ahead energy market for the same time period, respectively.

# C. Steps for generating wind power output scenarios

This paper adopts the wind power output scenario generation method proposed in Ref. 5. Using this method, scenarios that incorporate the information of forecast error distribution and fluctuation distribution for short-term wind power can be generated. The specific steps are detailed below:

S1) Fit the forecast error distribution function

First, we statistically analyzed the day-ahead forecast and actual output values of wind power within Ireland's power system in 2020 6, and converted the relevant data to the required scale (100 MW). Next, the wind power output forecast data were sorted. The range of 0-100 MW was divided into 20 power intervals (corresponding to 20 forecast bins, each with a width of 5 MW). The empirical cumulative distribution function was used to describe the forecast error distribution for wind power in each bin. For a given value x, the empirical cumulative distribution function is derived by summing the probabilities of all observations in the dataset that are less than x.

S2) Calculate the covariance matrix

This paper utilizes the covariance matrix G to reflect the correlation of wind power forecast errors across different time intervals, ensuring that randomly generated wind power output scenarios align with the statistical patterns of wind power fluctuations. The exponential covariance function 7 is employed to calculate the covariance.

S3) Generate wind power output scenarios

After obtaining the covariance matrix G, S random vectors Z are generated, following a multivariate normal distribution N(u0, G), where u0 is a T-dimensional zero vector. By applying the inverse transformation to the random vectors Z, wind power forecast error scenarios can be generated. These forecast error scenarios are then added to the predicted values to produce S wind power output scenarios.

Subsequently, the K-means clustering method is used for scenario reduction, yielding six wind power output scenarios as shown in Fig. 15.

# D. Linearization methods for equations

Linearization methods for Eqs. (17) and (20) in the main text are provided below. Equations (17) and (20) are as follows:

(S17)

(S18)

Taking a specific t,s as an example, the linearization method for Ct,sw,tcl in Eq. (S17) is provided as follows:

1) Define A=Ct,sw,tcl, B=Pt,sw2tcl,rt, r=ctw2tclΔt, h=Lttcl,con, m=Lttcl,1, n=Lttcl,max, then Eq. (S17) can be written as:

(S19)

2) Equation (S19) can be rewritten as:

(S20)

3) Define C=min(B, m), then Eq. (S20) can be written as:

(S21)

4) Then linearize C:

(S22)

where M is a sufficiently large number.

Taking a specific t,s as an example, the linearization method for Pt,se,dev,sup in Eq. (S18) is provided as follows:

1) Define A=Pt,se,dev,sup, B=Pt,se,rt, C=(1+αt)Pi,tws then the original equation can be written as:

(S23)

2) Then the linearized expression is as follows:

(S24)

where M is a sufficiently large number. Pt,se,dev,inscan be linearized in a similar method to the process described above.

# References

1M. Song, D. Li, C. Liu, Y. Li, X. Liu, and M. Zeng, "Comprehensive Benefit Evaluation of Combined Operation of Wind Power and Electric Heating," in 2021 IEEE IAS Industrial and Commercial Power System Asia, 2021, IEEE, pp. 1265-1271.

2W. Li, F. Dong, Z. Ji, and L. Ji, “Evaluation of provincial power supply reliability with high penetration of renewable energy based on combination weighting of game theory-TOPSIS method,” Sustainable Energy, Grids and Networks 35, 101092 (2023).

3W. Zha., H. Zhong., and Y. Liu., “Fair and Efficient Profit Allocation for Collaborative Operation of Distributed Renewable Energy Operators and Electric Vehicle Charging Stations,” J. Mod. Power Syst. Clean Energy, 1-13 (2024).

4Elia Group. see https://www.elia.be/en/grid-data/Load-and-Load-Forecasts for“Total load,” (last accessed March 06 2026).

5X. Ma, Y. Sun, and H. Fang, “Scenario generation of wind power based on statistical uncertainty and variability,” Ieee Trans. Sustain. Energy 4(4), 894-904 (2013).

6Eirgrid Group, see https://www.eirgrid.ie/grid/system-and-renewable-data-reports for“Annual renewable report”(last accessed March 06 2026).

7P. Pinson and R. Girard, “Evaluating the quality of scenarios of short-term wind power generation,” Appl. Energy 96, 12-20 (2012).